# Overview

This document defines the MQTT communication interface between the Switch device and the Management Platform.

# MQTT Topics

The following MQTT topics are defined:

switch/<sn>/first_seen        (Switch → Platform) Device onboarding / discovery

switch/<sn>/request          (Platform → Switch) Send command requests

switch/<sn>/response         (Switch → Platform) Command execution results

switch/<sn>/event	            (Switch → Platform) Device notifications / alarms

switch/<sn>/status            (Switch → Platform) Device state updates

switch/<sn>/telemetry         (Switch → Platform) Periodic statistics

switch/<sn>/file_transfer       (Platform → Switch) Firmware or configuration transfer

Switch/<sn>/process			 (Switch → Platform)Report the execution progress of 									file_transfer

switch/all/request             (Platform → All Switches) Broadcast command

# Status Code Definition

0 = Success

1 = Invalid command

2 = Invalid parameters

3 = Command execution failed

4 = Device busy

5 = Internal device error

# Device Onboarding (First Seen)

When a switch connects to the MQTT broker for the first time or boots up, it should publish device information.

Topic:

switch /<sn>/first_seen

Purpose:

- Allow automatic device discovery

- Store device information in platform database

- Enable onboarding workflow in UI

# Example First Seen Payload

{

"serial_number": "SN123456789",

"mac_address": "00:11:22:33:44:55",

"model": "TR-SW-24G",

"hardware_version": "1.0",

"software_version": "2.3.1",

"ip_address": "192.168.1.10/24",

"timestamp": 1710000000

}

Note:

"ip_address" - represents the IPv4 address/mask of the connection to the broker.

# Command Request Format / Command Response Format

- Reference documents

# Command Request Topic

switch /<sn>/request

Payload structure:

{

"request_id": "string",

"command": "string",

"params": {

}

}

Field Description:

request_id: Unique request identifier

command: Command name

params: Command parameters

# Example Request

{

"request_id": "req_1001",

"command": "show version",

"params": {

}

}

# Command Response Topic

switch /<sn>/response

Payload structure:

{

"request_id": "string",

"status": 0,

"message": " string ",

"data": "string"

}

# Example Response

{

"request_id": "req_1001",

"status": 0,

"message": " Success",

"data": " Software Version: xxxx \n Release Date: 2026-02-26 22:41:27\n System MAC Address: 70:98:cd:bb:98:50\n PID: xxxx , Hardware Version: 1.0, Serial Number: xxxx \n Vendor oid: xxxx \n "

}

# Broadcast Command

Topic:

switch/all/request

Purpose:

Send a command to all switches simultaneously.

All switches should subscribe to this topic and execute the command when received.

The " command " is consistent with the description in the Command Request Format.

# Event Notification

Topic:

switch /<sn> /event

Events represent asynchronous notifications generated by the switch.

Examples:

- port down

- port up

# Example Event Payload

{

"event": " port_ up ",

"timestamp": 1710000000,

"data": {

"port": "eth1"

}

}

{

"event": "port_down",

"timestamp": 1710000000,

"data": {

"port": "eth1"

}

}

# Device Status Reporting

Topic:

switch /<sn>/status

Used to report device lifecycle state such as online /offline or rebooting .

# Example Status Payload

{

"status": "online",

"uptime": 123456,

"software_version": "2.3.1"

}

Note:

- Offline implementation relies on the MQTT LWT ( Last Will and Testament ) mechanism, with an uptime of zero.

- Rebooting cannot guarantee that the notification message will be delivered.

# Telemetry Reporting

Topic:

switch /<sn>/telemetry

The switch should periodically publish telemetry information (recommended interval: 30 seconds). We're considering whether a 30 seconds interval is too short, as it could put continuous pressure on the platform. Therefore, we've added a configurable command: "mqtt client report-interval <10-65535>". What should the default value be?

Telemetry includes system metrics and interface metrics.

# Telemetry System Metrics

Required system telemetry fields:

- cpu_usage

- memory_usage

- temperature

- serial_number

- software_version

# Telemetry Interface Metrics

For each interface (port), the following information should be provided:

- port

- status

- mac_address

- ip_address

- rx_bytes

- tx_bytes

- rx_packets

- tx_packets

# Example Telemetry Payload

{

"timestamp": 1710000000,

"cpu_usage": 15,

"memory_usage": 40,

"Temperature": 45

"serial_number": "SN123456789",

"software_version": "2.3.1",

"interfaces": [

{

"port": "eth0",

"status": "up",

"mac_address": "AA:BB:CC:DD:EE:01",

"ip_address": "192.168.1.10/24",

"rx_bytes": 123456,

"tx_bytes": 654321 ,

"rx_packets": 12345678,

"rx_packets": 87654321

}

]

}

Note:

"cpu_usage": 15  - indicates that CPU usage is 15%.

"memory_usage": 40  - indicates that memory usage is 40%

JSON uses the double type as the numeric data type, while rx_bytes, tx_bytes, rx_packets, and tx_packets are 64-bit unsigned integers, which may lead to out-of-bounds errors. Furthermore, these statistics are only displayed for physical ports; they are not displayed for SVI ports.

# File Transfer

Topic:

switch /<sn> /file_transfer

Used to initiate firmware updates, configuration downloads, or other file -based operations.

The device should download the file from the provided URL and verify checksum before applying it.

Algorithm supports :

MD5/SHA1/SHA256/SHA512

file_type support:

Firmware : Firmware upgrade

Backup_configuration: Backup configuration

Restore_configuration: Change configuration

Restore_certificate: Change certificate

Restore_privateKey: Change the key

Restore_ca: Change the ca
Note：
Firmware and Restore_configuration require a device reboot to take effect.

Restore_ca, Restore_certificate and Restore_privateKey require MQTT to be re-enabled.

# Example File Transfer Payload

{

"request_id": "file_001",

"file_type": "firmware",

"file_url": "https://server/files/fw_v2.1.bin",

" algorithm ": " MD5 ",

"checksum": "9f3a7c2d",

"version": "2.1"

}

Note:

During file operations, it is necessary to prevent restart operations.

Do we need a function to return the progress, indicating that the operation is still ongoing?
To address this, we've added the `switch/<sn>/process` topic to provide real-time updates on file execution status, such as `Transferring/Verifying/Applying`. Furthermore, `Transferring/Applying` will continue to update `process` after a `report-interval`.

# Example File Process Payload

{

"request_id": "file_001",

"status": 0,

"message": "Success",

"data": {

"state": "Verifying"

}

}

# Example success Response

file_type : Firmware / Restore_configuration / Restore_certificate / Restore_privateKey

{

"request_id": "file_001",

"status": 0,

"message": " Success",

"data": {

}

}

file_type : Backup_configuration

{

"request_id": "file_001",

"status": 0,

"message": " Success",

"data": {

"algorithm": "MD5",

"checksum" : "9f3a7c2d"

}

}

# Example failed response

timeout:

{

"request_id": "file_001",

"status": 3 ,

"message": "Command execution failed",

"data": {

}

}

Firmware error or insufficient memory:

{

"request_id": "file_001",

"status": 5 ,

"message": "Internal device error",

"data": {

}

}

Checksum error:

{

"request_id": "file_001",

"status": 3 ,

"message": "Invalid parameters",

"data": {

}

}

# Necessary configuration

Note:

- Please synchronize your clock first to prevent issues arising outside the certificate's effective timeframe.

- "commander" is the alias we're using for now; you can provide a better one later.

- The upgraded certificate will be lost after a reboot; we will make it permanent in a future version.

SWITCH#upgrade mqtt-ca URL

SWITCH#upgrade mqtt-client-cert URL
SWITCH#upgrade mqtt-client-key URL

SWITCH(config)#mqtt client server URL

SWITCH(config)#mqtt client scheme commander

SWITCH(config)#mqtt client enable